CAPSTONE PROJECT

Complete Research & Execution Plan

Threshold Sensitivity Analysis of Circuit Breaker Configurations

on Cascading Failure Blast Radius in a Cloud-Native Spring Boot Microservice Mesh

with ML-Driven Adaptive Configuration via AWS-Native Services

B.Tech Computer Engineering  |  NMIMS Navi Mumbai  |  2024–25

Capstone Project — 8th Semester

# Part A — What We Are Building

This project is not just a Spring Boot microservice application. It is a full-scope distributed systems research initiative that produces actionable, empirically validated outputs for cloud reliability engineering.

## A.1  Project Identity

The project spans five simultaneous engineering and research disciplines:

- Distributed Systems Research — studies how distributed failures propagate across service chains

- Cloud-Native Reliability Engineering Platform — deployed on AWS ECS Fargate with real managed service dependencies

- Chaos Engineering Experimentation Framework — AWS FIS and Toxiproxy for controlled, repeatable fault injection

- Resilience Optimization System — evaluates Resilience4j configuration parameters against blast radius outcomes

- ML-Assisted Configuration Recommendation Tool — trains on experiment data to recommend optimal configurations pre-deployment

## A.2  Core Deliverables

| Deliverable | Format | Description |
| --- | --- | --- |
| Parameter Sensitivity Heatmap | Grafana dashboard + PNG | Shows how CB parameters affect blast radius across all fault modes |
| ML Recommendation Engine | Lambda REST endpoint | Decision Tree trained on experiment runs — maps fault profile to optimal config |
| Cloud-Native Fault Injection Platform | GitHub repo + CDK stack | Reusable experimentation framework for ECS Fargate |
| Research Paper | IEEE-format PDF | Full empirical paper with literature review, methodology, results, novelty analysis |

## A.3  One-Line Research Summary

This project experimentally studies how Resilience4j circuit breaker configurations influence cascading failures in distributed Spring Boot microservices under real cloud-native fault conditions, and produces a data-driven recommendation framework for resilience optimization — the first such empirical study at the Resilience4j application layer.

# Part B — Research Strategy

## B.1  The Core Problem

Modern distributed systems rely on circuit breakers as the primary defence against cascading failures. Three structural problems exist in how teams use Resilience4j:

- Most developers deploy with default configurations — never tuning to their specific traffic or failure profile

- Circuit breaker tuning is reactive — teams calibrate after incidents, not before them

- No empirical guidance exists — no dataset, heatmap, or decision tool maps configuration parameters to blast radius outcomes

A threshold optimised after a latency spike may be actively wrong during an Availability Zone partition or managed service throttle event. This is single-mode calibration: locally rational, globally miscalibrated.

## B.2  Research Questions

| RQ | Research Question | Gap Addressed |
| --- | --- | --- |
| RQ1 | How do Resilience4j CB parameters influence cascading failure blast radius in multi-service Spring Boot on AWS ECS Fargate? | No Resilience4j parameter sweep across a service chain |
| RQ2 | Which parameter contributes most to blast radius reduction under different cloud-native failure modes? | No failure-mode stratification at the parameter level |
| RQ3 | How do COUNT_BASED and TIME_BASED sliding windows differ in blast radius outcomes under transient vs sustained fault injection? | No empirical COUNT vs TIME comparison exists anywhere |
| RQ4 | What trade-offs exist between isolation speed, recovery time, false positive rate, and throughput loss? | Blast radius and trade-off dimensions unmeasured |
| RQ5 | Can experimental sensitivity data train a model that recommends optimal configs given a fault class and traffic profile? | No pre-deployment decision tool exists |

## B.3  Independent & Dependent Variables

| Variable | Type | Values Tested |
| --- | --- | --- |
| failureRateThreshold | Independent | 30%, 50%, 70% |
| slidingWindowSize | Independent | 5, 10, 20 requests |
| waitDurationInOpenState | Independent | 5s, 15s, 30s |
| slidingWindowType | Independent | COUNT_BASED, TIME_BASED |
| faultType | Independent | Latency, Crash, Throttle |
| trafficPattern | Independent | Bursty, Sustained |
| topologyType | Independent | Linear, Fan-Out, Shared Dependency |
| Blast Radius | Primary Dependent | % services in OPEN or degraded state at peak fault |
| Time-to-Open | Secondary Dependent | Seconds from fault onset to circuit opening |
| Time-to-Recover | Secondary Dependent | Seconds from fault resolution to CLOSED state |
| False Positive Rate | Secondary Dependent | % unnecessary circuit openings during healthy traffic |
| Client Error Rate | Secondary Dependent | % requests returning 5xx at ALB |
| Throughput Loss | Secondary Dependent | % drop in successful TPS vs baseline |

# Part C — Multi-Topology Strategy

Topology is now a formal experimental variable. Three predefined archetypes replace the original single linear chain. This is the most significant evolution from the original plan.

## C.1  Why Multi-Topology

The original single-topology design had strong causal clarity but a critical ML weakness — a Decision Tree trained on one fixed topology structure is open to the criticism: "This just memorised one mesh, it hasn’t learned anything transferable." The multi-topology strategy closes that hole entirely.

| Dimension | Single-Topology (Original) | Multi-Topology (New) | Winner |
| --- | --- | --- | --- |
| Causal clarity | Very high — one path, clean attribution | Moderate — more variables | Original |
| ML feature richness | Low — one structural context | High — topology features added | New ↑↑ |
| ML generalisability | Weak — overfits to one topology | Stronger — learns across archetypes | New ↑↑ |
| Industrial realism | Limited — linear chains are rare in prod | High — fan-out and shared DB are common | New ↑↑ |
| Research novelty | High on CB params, limited on topology | Higher — topology × CB config is unexplored | New ↑ |
| Implementation complexity | Lower — one CDK stack | Higher — 3 topologies, 3 CDK configs | Original |
| SRE applicability | Moderate — context-narrow | High — topology-specific tuning actionable | New ↑↑ |

## C.2  The Three Topology Archetypes

### Topology 1 — Linear Chain (Primary Baseline)

Gateway → Order Service → Inventory Service → Payment Service → Notification Service

Role: Primary dataset generation, main experimental conclusions, core blast radius analysis, ML baseline training. This topology must be completed before touching topologies 2 or 3.

| Scientific Benefit | Reason |
| --- | --- |
| Clean propagation | Easier causal analysis — failures move in one direction |
| Simple dependencies | Better reproducibility across experiment batches |
| Clear blast radius | Easier measurement — propagation depth is unambiguous |
| Strong internal validity | Fewer uncontrolled variables than mesh topologies |

### Topology 2 — Fan-Out / Tree

Gateway / ¦ \ / ¦ \ Order Inventory Payment

Role: Parallel dependency analysis, failure amplification studies, concurrent service degradation. Used for comparative validation after Topology 1 is complete.

Reveals how failures spread across parallel branches simultaneously, how retry storms amplify blast radius when multiple callers hit the same upstream fault, and how COUNT_BASED windows behave differently when fan-out saturates them faster.

### Topology 3 — Shared Dependency Mesh

Order Service → RDS Inventory Service → RDS Payment Service → RDS

Role: Bottleneck analysis, shared dependency failures, contention propagation. Used for stress topology analysis after Topology 2 comparative runs.

Simulates the most common production failure pattern — database overload affecting many services simultaneously, creating correlated failures, cascading contention, and retry amplification. This is the topology most directly relevant to RDS throttle fault injection.

## C.3  Experiment Count Comparison

| Design | Calculation | Total Runs |
| --- | --- | --- |
| Original (single topology) | 27 configs × 3 faults × 6 param combos × 3 replications | 486 runs |
| New (multi-topology) | Same per topology × 3 topologies | 486–729 runs |
| Practical approach | Full sweep on Topology 1, comparative on 2 & 3 | ~600 runs total |

Topology 2 and 3 do not require the full 486-run sweep. They run representative configurations from the Topology 1 sensitivity analysis — the top and bottom performers by blast radius — to validate whether topology structure changes the ranking. This is comparative analysis, not a full replication.

# Part D — Risk Management & Result Divergence

## D.1  The Divergence Problem

Controlled experiments produce results under laboratory-clean conditions — fixed load, fixed fault injection timing, fixed topology wiring. Production cloud experiments introduce variables that cannot be fully controlled:

- ECS task scheduling jitter — Fargate doesn’t guarantee when a task restarts after FIS termination

- Network nondeterminism — real AWS VPC routing introduces variable latency that Toxiproxy doesn’t replicate perfectly

- JVM warmup effects — Spring Boot services behave differently in the first 30–60 seconds vs steady state

- Shared infrastructure noise — RDS connection pool behaviour changes under real CloudWatch-triggered throttle vs simulated throttle

- Gatling load variance — actual TPS achieved vs configured TPS drifts under real network conditions

## D.2  Re-Run Cost If Divergence Is Detected Late

| Scenario | Runs Affected | Estimated Re-Run Time |
| --- | --- | --- |
| One fault class invalid (e.g. latency class) | ~162 runs | 8–10 days |
| All three fault classes invalid | 486 runs | 3–4 weeks |
| One topology’s results invalid | 243 runs | 10–14 days |
| Single config family invalid | 27–54 runs | 1–2 days |

A full 486-run re-run late in the semester is a project-killing event. The mitigations below prevent it — none of them are optional.

## D.3  Divergence Mitigations (All Mandatory)

### Mitigation 1 — Canary Runs Before Full Sweep

Before committing to any full fault-class sweep on AWS, run 15–20 representative configurations and compare blast radius readings against controlled results. If divergence is within ±15%, proceed. If larger, diagnose and fix before running all 162 configs in that fault class.

Cost of a canary: 20 runs × 10 minutes = ~3 hours. Cost of catching a problem here vs after 162 runs: 10 days saved.

### Mitigation 2 — Declare an Acceptable Divergence Band

In the methodology section of the research paper, explicitly declare that controlled and production results are expected to differ by up to ±15% in blast radius due to infrastructure nondeterminism. This is academically honest and means a 12% divergence is not a failure — it is a documented and expected observation. Only divergences outside this band trigger investigation.

### Mitigation 3 — Priority Ordering of Experiment Runs

Run extreme configurations first — the most aggressive threshold settings and the most conservative ones. These produce the most distinctive blast radius signals. If production and controlled results agree on the extremes, the middle configurations are unlikely to diverge badly enough to matter. Run them in this order:

- failureRateThreshold=30, slidingWindowSize=5, waitDuration=5s (most aggressive)

- failureRateThreshold=70, slidingWindowSize=20, waitDuration=30s (most conservative)

- All intermediate configurations

### Mitigation 4 — Freeze the Controlled Environment

Once local Docker Compose controlled runs are complete, lock that environment. No dependency updates, no Toxiproxy version changes, no Gatling script modifications. If AWS production runs diverge, the controlled environment can be re-run in hours because it is stable and reproducible.

# After controlled runs complete, pin all versions docker-compose.yml: lock all image tags to exact digests pom.xml: set <resilience4j.version> to exact patch version requirements.txt: pin scikit-learn==1.4.0 exactly Gatling: commit simulation files, do not modify after baseline runs

### Mitigation 5 — Treat Divergence as a Finding

If certain configurations behave differently in production vs controlled environments, that gap is itself a research contribution. It tells you which circuit breaker configurations are sensitive to infrastructure nondeterminism and which are robust. This goes into the results section as a validity analysis, not into the bin.

## D.4  Full Risk Register

| Risk | Severity | Probability | Mitigation |
| --- | --- | --- | --- |
| Result divergence beyond ±15% in one fault class | High | Medium | Canary runs before full sweep; divergence band declared in methodology |
| Implementation time blowout across 3 topologies | High | Medium | Topology 1 hard gate; topologies 2 & 3 only after primary dataset complete |
| Dataset inconsistency across topologies | Medium | Low | Identical fault injection scripts and Gatling profiles across all topologies, locked early |
| ML model overfits on small per-topology dataset | Medium | Medium | Keep Decision Tree max_depth ≤6; 5-fold cross-validation; report train vs test accuracy |
| Claiming universal topology generalisation | High | Low | Frame as ‘representative archetypes’ explicitly in paper; never claim universal generalisation |
| AWS cost overrun (3 meshes on Fargate) | Medium | Low | Tear down after each batch; Fargate Spot; topologies 2 & 3 run comparative sets only |
| JVM memory pressure corrupting local results | Medium | Medium | Cap all services at -Xmx512m in Docker Compose JAVA_OPTS |
| FIS experiment timing inconsistency | Low | Medium | Run each config 3 times; use median blast radius; discard outliers beyond 2 standard deviations |
| COUNT vs TIME window novelty diluted by topology complexity | Low | Low | Every topology run must include both window types; this dimension is never dropped |

# Part E — System Requirements

You do not need one powerful machine. You need the right hardware at the right phase. Three distinct environments serve three distinct purposes.

## E.1  Environment 1 — Local Development Machine

This is where all controlled experiments run. It never touches AWS during Phase 1–4. All six Spring Boot services run in Docker Compose, Toxiproxy injects faults, Prometheus and Grafana collect and visualise results.

| Component | Minimum | Recommended | Why It Matters |
| --- | --- | --- | --- |
| CPU | 8 cores (i5 / Ryzen 5) | 12–16 cores (i7/i9 / Ryzen 7) | Gatling + 6 JVMs competing for CPU; artificial throttling on fewer cores corrupts results |
| RAM | 16 GB | 32 GB | 6 JVMs + Prometheus + Grafana + Postgres + Gatling = 10–12 GB active; 16 GB is tight |
| Storage | 50 GB SSD | 100 GB NVMe SSD | Docker images, experiment result JSON, Prometheus TSDB |
| OS | Ubuntu 22.04 / macOS / WSL2 | Ubuntu 22.04 native | Docker Engine on native Linux outperforms Docker Desktop by 20–30% |
| Docker | Docker Desktop | Docker Engine (native) | Native engine has lower overhead; critical during load test phases |

RAM is the critical constraint. Six Spring Boot services × 512 MB JVM heap = 3 GB minimum for application layer alone. Add Prometheus (500 MB), Grafana (300 MB), PostgreSQL (400 MB), DynamoDB Local (300 MB), and Gatling (1 GB under load) = ~5.5 GB overhead. On 16 GB you will be tight. On 32 GB you will be comfortable. Fix: always set -Xmx512m -Xms256m in JAVA_OPTS for every service in docker-compose.yml.

# docker-compose.yml — JVM heap cap for all services services: order-service: environment: JAVA_OPTS: "-Xmx512m -Xms256m -XX:+UseG1GC" payment-service: environment: JAVA_OPTS: "-Xmx512m -Xms256m -XX:+UseG1GC" # repeat for all 6 services

What runs locally:

- All six Spring Boot services (Docker Compose)

- Toxiproxy sidecar for latency injection

- Local Prometheus + Grafana for dashboards

- Local PostgreSQL + DynamoDB Local (no AWS DB costs)

- Gatling 3.10 load generator

- Python experiment runner (automates all 486 local runs)

- scikit-learn model training — Isolation Forest + Decision Tree (runs in seconds on any CPU)

## E.2  Environment 2 — AWS Cloud

Not a machine you own — a set of managed services spun up per experiment batch and torn down after. You pay only for what you use. AWS provides capabilities that the local environment fundamentally cannot replicate:

- Real ECS Fargate task termination via FIS — cannot be simulated in Docker

- Real RDS connection throttling — DynamoDB Local does not replicate throttle behaviour

- Real network partitions between availability zones

- X-Ray distributed tracing across actual managed infrastructure

- CloudWatch metric streams feeding the Lambda ML pipeline

| AWS Service | Role | When Active | Cost Tier |
| --- | --- | --- | --- |
| ECS Fargate (6 tasks) | Runs all six Spring Boot services | Experiment batches only — tear down after | Paid — use Fargate Spot (70% saving) |
| AWS FIS | Fault injection engine | Only during individual experiment runs | Paid per action |
| RDS db.t3.micro | Order + Inventory persistence | Experiment batches only — pause when idle | Free tier (750 hrs/mo) |
| DynamoDB on-demand | Payment transaction log | Always on | Mostly free (25 GB + 25 WCU free tier) |
| Application Load Balancer | Entry point, error rate measurement | Experiment batches only | Paid (LCU pricing) |
| AWS Lambda | ML inference + anomaly detection pipeline | Always on (serverless) | Mostly free (1M req/mo free tier) |
| S3 | Stores all experiment result JSON | Always on | Very cheap — experiment data is small |
| CloudWatch + X-Ray | Metrics and distributed tracing | Active during experiments | Limited free tier |
| AWS Parameter Store | Stores Resilience4j config per service | Always on | Free (standard tier) |
| Amazon SNS | Batch completion alerts | Always on | Free (1M publishes/mo) |

Estimated total AWS cost for the full experiment matrix (486 primary runs + 150 comparative runs across topologies 2 & 3) using Fargate Spot and disciplined batch teardown: approximately ₹3,500–₹6,500 (USD 40–80). The RDS instance stays within free tier if paused between batches.

## E.3  Environment 3 — Experiment Orchestrator

Something must trigger experiment runs, collect results, push to S3, and notify when a batch completes. This does not need to be powerful.

| Option | Cost | Recommendation |
| --- | --- | --- |
| EC2 t3.micro | Free tier eligible | Best option — stays alive between batches, runs Python orchestrator persistently, sends SNS notifications |
| GitHub Actions free tier | Free (2000 min/mo) | Adequate for triggering CDK deployments and short orchestration scripts |
| Local machine | Free | Works if you are willing to leave it running; not ideal for overnight batch runs |

## E.4  Full Environment Architecture

Your Laptop (32 GB RAM, 12+ cores) │ ├── Weeks 1–4 (Controlled Phase): Everything runs here │ ├── Docker Compose — 6 Spring Boot services │ ├── Toxiproxy sidecar │ ├── Prometheus + Grafana │ ├── PostgreSQL + DynamoDB Local │ ├── Gatling load generator │ └── Python experiment runner (486 local runs) │ └── Weeks 5–8 (Cloud Phase): Orchestrates AWS from here │ └── AWS Cloud ├── ECS Fargate (6 tasks) ← spin up / tear down per batch ├── RDS db.t3.micro ← spin up / tear down per batch ├── AWS FIS ← active only during fault window ├── DynamoDB on-demand ← always on ├── Lambda ML pipeline ← always on, serverless ├── S3 (experiment results) ← always on ├── CloudWatch + X-Ray ← active during experiments └── EC2 t3.micro orchestrator ← always on, free tier

## E.5  What You Do NOT Need

| Not Needed | Why |
| --- | --- |
| Dedicated server or lab machine | Laptop handles the entire controlled phase |
| GPU | Decision Tree and Isolation Forest on 486 rows runs in seconds on any CPU |
| Kubernetes cluster | ECS Fargate manages orchestration; no k8s management needed |
| High-end cloud instances | db.t3.micro and Fargate 0.5 vCPU / 1 GB tasks are sufficient for controlled experiment load |
| Persistent cloud infrastructure | Everything expensive is ephemeral; spun up per batch, torn down after |
| Grafana Cloud or Managed Grafana | Local Grafana on EC2 t2.micro (free tier) serves the dashboard |

# Part F — Execution Plan

Topology 1 is a hard gate. The full 486-run sweep, heatmap, and baseline ML model on the linear topology must be complete before a single line of CDK is written for topology 2. At that point all core deliverables are protected.

## F.1  Phase Timeline

| Phase | Weeks | Environment | Goal | Gate to Next Phase |
| --- | --- | --- | --- | --- |
| 1 — Local build | 1–2 | Local Docker | All 6 services running, Resilience4j wired, Prometheus scraping | All services healthy under Gatling baseline load |
| 2 — Fault injection | 3–4 | Local Docker | Toxiproxy latency, service crash, DB throttle working; blast radius metric live | All 3 fault classes reproducible locally |
| 3 — Controlled experiments | 4–5 | Local Docker | Full 486-run sweep on Topology 1 complete; heatmap generated | Heatmap shows clear parameter sensitivity signal |
| 4 — ML training | 5 | Local Python | Isolation Forest anomaly detector trained; Decision Tree recommender trained on local results | Both models produce coherent predictions |
| 5 — AWS canary | 6 | AWS (limited) | 15–20 canary runs on AWS; divergence from local results within ±15% | Canary divergence acceptable; proceed to full cloud sweep |
| 6 — AWS primary sweep | 6–7 | AWS | Full 486-run Topology 1 sweep on AWS; results stored in S3; heatmap updated | AWS and local heatmaps substantively consistent |
| 7 — Topology 2 & 3 | 7–8 | AWS | Comparative runs on Fan-Out and Shared Dependency topologies; ML model retrained with topology features | Topology sensitivity findings documented |
| 8 — Paper & deliverables | 8 | Local | IEEE-format research paper; final heatmap dashboard; Lambda recommender deployed | All 5 deliverables complete |

## F.2  Experiment Execution Order Within Each Phase

Within each fault class, run configurations in this order to enable early divergence detection:

- Extreme aggressive config (threshold=30, window=5, wait=5s) — both COUNT and TIME

- Extreme conservative config (threshold=70, window=20, wait=30s) — both COUNT and TIME

- If extremes agree with controlled results, run all intermediate configurations

- If extremes diverge, stop, diagnose, fix, and re-run extremes before proceeding

## F.3  ML Component Execution

### Stage 1 — Runtime Anomaly Detection (Isolation Forest)

| Input Feature | Source | Signal |
| --- | --- | --- |
| error_rate_5s | Resilience4j Micrometer | % failed calls in last 5s per service |
| p99_latency_ms | AWS X-Ray trace aggregation | Tail latency — first signal of upstream degradation |
| circuit_state | Resilience4j events | 0=CLOSED, 1=HALF_OPEN, 2=OPEN |
| db_connection_wait_ms | RDS Enhanced Monitoring | Connection pool pressure — precursor to RDS fault cascade |
| sqs_queue_depth | CloudWatch SQS metrics | Async backlog indicator |

### Stage 2 — Pre-Deployment Recommender (Decision Tree)

| Input Features | Output Predictions |
| --- | --- |
| Fault class (latency / crash / throttle) | Recommended failureRateThreshold |
| Expected request rate (TPS) | Recommended slidingWindowSize |
| Recovery SLA target (seconds) | Recommended waitDurationInOpenState |
| Traffic pattern (bursty / sustained) | Recommended slidingWindowType |
| Topology type (linear / fan-out / shared) | Confidence score per recommendation |

Why Decision Tree and not deep learning:

- 486 rows — deep learning would overfit immediately

- Tabular structured data — exactly the domain where Decision Trees excel

- Operational teams need to understand WHY a config is recommended — Decision Trees provide feature importance

- The novel contribution is the training dataset, not model architecture

## F.4  Open-Source Stack

| Component | Version | Role | Cost |
| --- | --- | --- | --- |
| Spring Boot | 3.2 | Six-service application framework | Free (Apache 2.0) |
| Resilience4j | 2.2 | Circuit breaker library — the study subject | Free (Apache 2.0) |
| Prometheus | Latest | Metrics scraping and storage | Free (Apache 2.0) |
| Grafana | 10 | Heatmap and experiment dashboards | Free (AGPL) |
| Gatling | 3.10 (Java DSL) | Load testing — simulates realistic traffic | Free (Apache 2.0) |
| Toxiproxy | 2.9 | Network-layer latency injection sidecar | Free (MIT) |
| Docker + Compose | Latest | Local orchestration | Free |
| AWS X-Ray SDK | Latest | Distributed tracing | Free SDK |
| Python + scikit-learn | 3.12 / 1.4 | ML model training | Free |
| AWS CDK (Java) | Latest | Infrastructure as Code | Free (Apache 2.0) |

# Part G — Novel Contributions & Academic Positioning

## G.1  Research Gaps Filled

| Gap | Status in Literature | This Project |
| --- | --- | --- |
| COUNT_BASED vs TIME_BASED sliding window empirical comparison | Absent across all reviewed papers — exists only in Resilience4j documentation | First empirical comparison under bursty vs sustained fault injection across 486 runs with both types |
| ML-driven adaptive Resilience4j configuration | Sedghpour 2023 proposes Istio-layer adaptation; zero papers at application-layer Resilience4j | Lambda + Isolation Forest + Parameter Store live update — first application-layer adaptive config |
| Failure-mode stratified parameter sensitivity | No paper crosses fault mode × circuit breaker config | Full 3×4 parameter matrix × 3 fault classes — first stratified sensitivity study |
| Blast radius as a primary numerically defined metric | 4 papers deploy multi-service architectures; none define or measure blast radius numerically | Blast radius instrumented via custom Actuator aggregator and X-Ray service maps |
| Pre-deployment decision tool | All recommendations are post-hoc observations; no callable tool produced | Heatmap + Decision Tree Lambda endpoint — explicit, reusable engineering artefact |
| Topology-aware circuit breaker sensitivity | Not studied anywhere | Multi-topology design adds topology characteristics as ML features and experimental variable |

## G.2  Correct Academic Framing

Do NOT claim:

This study generalises across all topologies and all production environments.

DO claim:

The study evaluates representative cloud-native topology archetypes under controlled fault conditions, producing the first empirical sensitivity dataset for Resilience4j circuit breaker configurations at the application layer.

## G.3  Final Research Positioning

A controlled multi-topology resilience engineering study evaluating how Resilience4j circuit breaker configurations influence cascading failure propagation across representative cloud-native dependency archetypes, with ML-driven adaptive configuration via AWS-native services — the first such empirical study at the Resilience4j application layer.

NMIMS Navi Mumbai  |  B.Tech Computer Engineering  |  2024–25